TED(21)–3035 REVISION2021

Reg.No.………..

Name:………….

DIPLOMAEXAMINATIONINENGINEERING/TECHNOLOGY/MANAGEMENT/ COMMERCIAL PRACTICE.

# MECHANICAL ENGINEERING

Time:3hours	MaximumMarks: 75

# PARTA

- Answer all the following questions in one word or sentence.

# (9x1=9Marks)

ModuleOutcomeCognitivelevel

| 1 | Specific gravity is the ratio of ………………. | M1.01 | R |
| --- | --- | --- | --- |
| 2 | Find out the weight densityof a fluid whose mass density is 2000kg/m3 | M1.04 | A |
| 3 | Write about irrotational flow of a fluid | M2.01 | R |
| 4 | Enumerate two assumption of Bernoulli’s theorem | M2.03 | R |
| 5 | Write example for impulse water turbine | M3.02 | R |
| 6 | Specific speed of turbine is ……………….. | M3.03 | R |
| 7 | Ports are used in ________ stroke engine | M4.04 | R |
| 8 | Compare IC engine and EC engine | M4.02 | R |
| 9 | ……………. Fuel is normally used in light duty vehicles | M4.03 | R |

- Answer any Eight questions from the following

# (8x3=24Marks)

ModuleOutcomeCognitivelevel

| 1 | Define the following terms - Relative density - Dynamic viscosity - Absolute pressure | M1.01 | U |
| --- | --- | --- | --- |
| 2 | A fluid of specific gravity 0.4 is flowing through a pipe with a pressure head of 40m. find out its internal pressure | M1.04 | A |
| 3 | Explain the concept of continuity and write continuity equation | M2.03 | U |
| 4 | A pipe of 50mm diameter is conveying water with a velocity of 2.5 m/s. find the loss of head at the entrance and exit of pipe (Take f= .005) | M2.04 | A |
| 5 | Explain water power and brake power of water turbine | M3.02 | U |
| 6 | Explain the components of a centrifugal pump with figure | M3.03 | U |
| 7 | Explain the selection on KW rating of various pumps based on head | M3.04 | U |
| 8 | Explain fire tube boiler | M4.01 | U |
| 9 | Explain the working of steam turbine | M4.02 | U |
| 10 | Compare two stroke and four stroke engine | M4.04 | U |

# PARTC

Answerallquestionsfromthefollowing. Each question carries seven marks

(6x7=42Marks)

ModuleOutcomeCognitivelevel

|  | Explain about inverted U-Tube manometer with figure OR A simple manometer containing mercury is used to measure the pressure of waterflowing in a pipe line. The mercury level in the open tube is 40mm higher than that on left tube. If the height of water in the left tube is 50mm. determine the pressure in the pipe | M1.04 | U |
| --- | --- | --- | --- |
|  |  | M1.04 | A |
|  | Explain losses through a pipe, and express Chezy’s and Darcy’s formula OR Explain the limitation and practical application of Bernoulli’s theorem | M2.03 | A |
|  |  | M2.04 | U |
|  | Using simple figure , explain the working of a hydroelectric power plant OR Using simple figure , write about Francis turbine | M3.02 | U |
|  |  | M3.02 | U |
|  | Explain the working of an air vessel OR Compare the types of water pumps | M3.03 | U |
|  |  | M3.03 | U |
|  | Explain theworking of impulse and reactionturbine OR Explain fire tube boiler with a line sketch | M4.02 | U |
|  |  | M4.01 | U |
|  | With neat figure explain the working of 4 stroke petrol engine OR With neat figure explain the parts of an IC engine | M4.04 | U |
|  |  | M4.04 | U |